Commentaries on NCREE South Branch-1

May 29, 2017

The following discussion starts to cover the on-site work that I have done since March, 2017 on the NCREE South Branch project installation. The items discussed will be the problems encountered and the temporary work-arounds and solutions to problems that have arisen. No comments are made about who, what, or why the problems occurred. Just the problems, solutions and results are explained. It should be noted that many of the problems are due to miscommunication or lack of attention to detail.

I arrived on February 26, 2017 in Taiwan to start my work on the NCREE South Branch project. My task was to perform the acceptance tests on the Shaking Table. However, there were to be several modifications to this plan.

Before I got into the 1st of many trials, I had to face a bigger one on the day of arrival. A science experiment had gone wrong on the shaking table 2 days before and had damaged the vertical actuator close-coupled accumulator mounts to the point where the table could not be operated. It was to take several weeks before the shaking table would be operational again. It was fortunate that initial installation of the table had been completed and valve-tuning and DOF tuning had taken place.

I then changed course and proceeded to learn as much about the system as I could to familiarize myself with the various parts of the project. There was, of course, a large shaking table in the facility that had yet to go through the major acceptance tests. It also had a serious problem in the vertical frequency response that had to be analyzed and solved before the table could operate over a wide frequency band. The vertical system response had a very deep hole at 16 Hz. It was suspected to be the static lift system, but there had not been any detailed tests made to determine the exact cause. It would continue to be a problem for some time to come. We could get through most of the acceptance tests because the frequency was not near any test frequency. Most of the tests were at frequencies like 5, 10, 20, and 30 Hz. The performance hole was not a major factor to these test frequencies. As improvements in frequency and performance were made, the “hole” was not a major factor in acceptance testing. More details on this problem later as we started table operation again.

The BATS (Bi-Axial Test System) is a bearing test machine that was supposed to be identical to the MATS (Multi-Axial Test System) that has been at NCREE North (Taipei) for about 10 years. This is the 4th bearing test system that I have participated in the installation. The other 2 were SRMD at USCD-San Diego and the BTS at Pavia University-Italy. I am the only person alive (Terry Nelson is deceased) that has any real operating experience on these machines outside of the local operating staff at these 4 sites. Therefore, I am responsible for the application software and operation of the BATS to get it commissioned and through acceptance tests as well. The BATS was intended to be quite similar to the MATS, but that turned out to not be the case. There are many mechanical differences between the MATS and BATS machines that required considerable MATS software changes to make it work on the BATS system. Details of these changes will be discussed later. At the time of 1st arrival, the BATS machine was still being mechanically installed. The actuators were not in place yet, so it would be some time before it would be useful for me to spend much time on its installation. The BATS Controller was not yet installed yet, either, but it would be installed during my 1st trip. I, of course, did use some time to familiarize myself with the new 494 BATS Controller. The MATS Controller is a 493 system and the E-Stop arrangement in South Branch is completely different than North Branch because of the shared hydraulic resources at NCREE South Branch.

There is also a Hydraulic Power Controller (HPC) that runs all of the 6 pumps in the South Branch facility and the blowdown accumulator system that supplies extra hydraulic power when required. This system has the further complication of having ball valves on each of the pumps to direct flow from each pump to one of 3 different places: shaking table, BATS, or Structural Strong Wall. These valves have switches on them so the HPC can sense where they are positioned. It was obvious that there were not enough pumps to allow independent operation of the 3 test operation centers. NCREE North does have a pumping system that had quite independent operation of the shaking table and structural operating centers. The addition of the MATS did upset this quite a bit, but additional pumps were added to the North Branch facility and some ball valves were added to allow the shaking table pumps to be used with the MATS on the rare occasion that it needed a large amount of flow. But, for the most part, NCREE North has independent operation of its 3 main operation centers. This is really not the case for NCREE South as previously discussed.

Commentaries on NCREE South Branch-1

May 29, 2017

I was not privy to any detailed information about the system, schedule, or specifications of the shaking table acceptance tests before my 1st arrival. I was to learn that there were some tests included in the acceptance test plan that were quite difficult to achieve. I question even why they were added because most customers would not even understand the reason. There was a requirement in the acceptance tests to perform Total Harmonic Distortion tests at 60% of maximum performance. At this high level, the THD of any waveform is dominated by the effect of the flow-velocity curve of the actuators. This results in predominantly 3rd-harmonic distortion in the range of 10-15%. This is nothing like the 25-100+% THD that can be found at lower test levels (5-10% of maximum performance). Some of these higher test levels required the use of the blowdown system to provide additional oil. These high levels could, therefore, not be sustained for long periods of time (minutes). Good THD measurements in the area of 5% cannot be done with data less than 2+ minutes in duration. These high level tests had some THD tests with less than 30 seconds of run time available. These tests were to push my abilities to the limit to get any decent results.

I left the site to get some time off for a few days. Another person came to start running the shaking table THD tests in my absence. He was able to get the low-level THD tests completed and some of the higher level THD tests done as well. There were some high-level THD tests that did not pass because of short test duration. It was now my turn to try to get good results. Operating the system was herky-jerky at the start because I had made some bad assumptions. I assumed that the E-Stops for the system had been tested. This was not the case. I would not make any further assumptions on-site here again. The 1st emergency occurred when the electric power breakers for the entire facility went out because they were set too low. Remember, I was now running at power levels on the pumps that were never done before at this facility. Usually, this power outage event fires an E-Stop. Not so this time. It seems that the HPC needs to have software limit detectors set to fire most E-Stops other than mechanical push buttons. There are, of course, limit detectors on the shaking table that were being used, but they do not include pumps stopping. There is software in the shaking table that usually detects an unusual pump stoppage, but it did not get any indication because the Wago PLCs in the pumps had no power. When they have no power, the Wago PLCs do not communicate and therefore, the pump status does not get updated.  And, of course, I had set only some displacement and force limits as per usual. Well, I added the required time-outs on the Wago PLCs in the pumps to the limit detector setup to tell me that they had stopped due to no electrical power. What had actually happened when we did not get an E-Stop for the pumps stopping is that we finally got an E-Stop from the 469D when the pilot pressure switches on the actuators fired. Of course, this E-Stop is too late to properly park the table or dump the static lift. All servo-control was suddenly lost to my complete surprise. There was a problem in the 469D software too that prevented the static lift to dump properly. Therefore, the table tipped up in the air due to excessive static lift and then fell against the park position. No major damage occurred, except to scrape off a table bumper that helps keep the table away from the foundation wall. The loss of servo-control on the loss of pilot pressure was a surprise because the main pressure was not too low yet. The main pressure check valve did not seem to work. As it turns out, there is a ball valve on that line as well as the check valve. It was closed. The ball valve was opened and the handle removed. The ball valve is there for maintenance reasons. (I know of no other system that has such a ball valve?).

Another side effect of this un-detected E-Stop was the discovery of a major change in the E-Stop reset function. In all systems prior to this one, the reset function was quite straight-forward. If an E-Stop occurred then the reset function could be accomplished by just pushing the reset button on the application GUI screen. Pushing that button then did everything required to restore the E-Stop chain. It seems now that this is not the case on this system. I was told that this was a “corporate” thing. I do not have to agree with this new arrangement, but I do have to live with it. It seems now that it can take many, many steps to get an E-Stop reset. If it is a local E-Stop on a given system, then the operator must:

- Push the GUI Reset button once, the application will not reset yet.

- Push the reset button on the local E-Stop box. The reset lamp on the box will light.

- Push the GUI Reset button on the application again to reset the application.

Commentaries on NCREE South Branch-1

May 29, 2017

This is bad enough, but if the Facility E-Stop button is pushed, then more trouble:

- Go to the HPC where-ever it is located and push its GUI reset button once. It will not reset yet.

- Push the HPC reset button on the HPC E-Stop box. The reset light will turn on.

- Push the HPC GUI reset button again.

- Return to the local controller and perform the 3-step reset process there again.

I personally think having to run to the HPC location and do that step on a Facility E-Stop is superfluous, but I have to live with it. It has caused many delays on-site because people (myself included) forget we have to do this step.

Because the 469D E-Stop did not dump the static lift system in time, this was a good time to review the entire static lift operation. I had difficulty in starting the system because of the static lift control software. The control software used a degree-of-freedom control system to control the static lift pressure. A table vertical force setpoint was the primary command for the static lift control software. There was also an activate button. The static lift control could not operate when the table is in the park position. There is no vertical force at that time. The static lift control is turned on when the table leaves the park position. Unfortunately, the servo-valves on the static lift were too small to charge the static lift cylinders with enough pressure to prevent cavitation before the table started to lift out of the park position. What I had to do is manually move the table out of park slightly to get the static lift to turn on with the table up just a few millimeters. I then had to wait until the controller had put enough oil into the lift cylinders so that they would not cavitate during the table lifting to the normal position. Why did I not just slow down the park to normal ramp rate? Well, then the table does not have enough time to park before we run out of oil from pumps being shut off by an E-Stop. The same ramp rate applies for both up and down. I manually moved the table to/from the Park position to prevent cavitation during the acceptance tests. To fix the problem, I requested to have the software changed to where the 4 static lift cylinders were charged to a given force while the table was still in the park position. This pre-charge would use the lift cylinder force as a feedback and not the usual table vertical force. During normal operation, the static lift control uses the actual vertical actuator force as the control feedback for the desired vertical force. The individual lift cylinder forces are then used to force balance the total lift force. The table vertical force is incorrect in the park position and cannot be used for any control. This new method required another parameter to be entered on the Static Lift Panel for this pre-lift cylinder charge force. The way this new control method worked is that the static lift controller would change to its normal operation at a certain table position away from park. This turned out to be an excellent solution. The reverse motion was not a problem for the static controller because the 4 servo-valves are jammed hard open to dump the lift cylinder pressure as soon as the operator selected Park on the 469D. The lift force reduction does take about 15-20 seconds to get below the table weight, so holding up a heavy specimen (240 Ton) is not a problem.

Concerning shaking table parking problems, we later encountered a problem where someone decided that the pumps were not dumping fast enough. He decided, on his own, to go to the pump room and turn all of the 10-turn dump needle valves full open. The pumps then dumped really fast. It was way too fast for any table parking motion to work right. It took quite a while to get it all back right. Some of us knew that 1-1.5 turns open is all that is required to get proper pump dump rates.

Commentaries on NCREE South Branch-1

May 29, 2017

Now, the acceptance tests for high level THD and maximum performance could be accomplished. The maximum performance tests were done first because I knew what had to be done for those tests.  The blowdown had been somewhat tuned and was working to some extent by the time I returned to the site. It was good enough to use, but was not optimized for any good performance. It had been used for some of the THD tests while I was gone. The maximum performance tests were to be performed for 2 test conditions: a) bare table and 2) 240 Ton specimen. At the last minute the customer added the 120 Ton specimen to the tests. This was accomplished by putting only 1 layer of concrete blocks on the table instead of the 2 layers for the 240 Ton specimen. All 3 were completely tested in 4 days total. The tests only took 3 hours for each specimen. The rest of the time was spent by the mechanical crew to put on/remove specimen(s). Luckily, the specimens were well designed and did not present any problems due to loosening of tie-down bolts. The tests go quickly because over the years, I have learned that these tests are the most likely to damage the table as any we do. Also, the table will never again in its lifetime be put to such extreme conditions. To that end, I make sure that at each sinusoidal test that we do the absolute minimum number of cycles at maximum performance. To accomplish this, I do the following:

- Create a spline sine curve start and end to a sine wave time history file that has only 2 to 6 cycles total at the maximum amplitude. The spline start/end ½ cycles ensure no nasty start/stop bangs due to bad acceleration/velocity/displacement steps. The few cycles limit the time at maximum performance. I use only 2 cycles at low frequency and 6 at high frequency because the high frequency response may actually take a cycle or two to get to maximum response, especially above the oil column natural frequency. I only need one cycle for maximum performance success.

- Run the sine time history using the 469D Data Player at about 20% amplitude. Adjust the span amplitude to get the proper response. This will minimize the number of runs needed at 100% amplitude. I never run any automatic amplitude control during these tests. The automatic control can run away too easily and cause serious problems. It also runs the table for too many cycles to even begin to work properly.

- Run again at 50% and adjust the span again if needed.

- Run the time history at 100%. Record the data with the Data Recorder.

- Make sure of a successful test by using the binview program to verify that a 100% response was achieved.

The following plot shows a sample time history for running maximum displacement performance tests. There are two ways to create this time history. One is with a MatLab program and the other is with a MatLab GUI exe file.

+/-1000 mm at 0.2 Hz

Only 1 cycle is required for proving the +/- 1 meter maximum performance.

Commentaries on NCREE South Branch-1

May 29, 2017

+/- 1.2 m/sec velocity at 1.0 Hz

This test is actually run in acceleration control at 1.0 Hz.

+/- 2.5 g’s at 10 Hz

This test uses more than one 100% cycle to be sure to get response on the 2nd cycle as shown. The same type of waveform is run on the 120 Ton and 240 Ton specimens to achieve maximum performance levels. All of the maximum performance tests passed.

Commentaries on NCREE South Branch-1

May 29, 2017

The 60% of maximum performance THD tests were more troublesome. The blowdown was required for several of the tests which limited the data collection time for good THD measurement. It took several runs at some of the frequencies to get “lucky” on the noise, etc. to make the specified THD. I had to remove the 60 Hz harmonic from some of the 30 Hz measurements because that harmonic was too big with electrical noise to pass. The THD specification says that 10 harmonics above the fundamental are to be summed for THD. Customers usually do not give us trouble with the power line frequency. I used a notch filter in binview to remove this harmonic, leaving the others intact. A table of the size at South Branch is typically considered to be a 0-30Hz system.

The problem with the poor vertical response at 16 Hz was proven to be an oil mass resonance in the static lift supply hoses. This problem did not show up in a similar design at South-West Ji Tong because that system had a smaller area and shorter stroke in the static lift cylinders. So, it was unfortunate that we ended up with a significantly larger amount of oil sloshing back and forth through the long rubber hose. To alleviate the problem, 1st, another hose was added (2 total) in parallel with the original hose and the trouble frequency went to 21 Hz and the response hole depth decreased. A 3rd hose was added in parallel and the frequency moved to 26 Hz and the depth improved to about a 35% drop in performance. A further test was proposed where an orifice should be introduced to attempt to dampen the mass flow rate.

This is where the work on this problem was suspended because the customer insisted on putting their own elastic specimen on the shaking table. This specimen has a lot of bolted connections that come loose with relatively small test levels. Even El Centro at 0.36 G was too much for this specimen. The variations in response from test run to test run was so bad that iteration methods could not improve the response very much at all. The repeatability was terrible.

Further work on the shaking table would have to wait until after the June government audit of the system. The NCREE people are working on the specimen to install their data acquisition equipment. NCREE purchased a FlexDAC system to go along with the shaking table system and future structural test equipment. The 2 problems that exist for this equipment are:

- There is no data synchronization of the FlexDAC with the shaking table system as of this date. No plan has been put in place because the scheduling has not been accomplished with the engineer that can do this work.

- The NCREE tri-axial accelerometers cannot be directly hooked up with the MTS FlexDAC system. The FlexDAC manual states that the conditioner excitation could source up to 50 ma of excitation current. Perhaps this is true if certain things are done to the FlexDAC, but, in the interim a solution had to be found because tests concluded that only about 25 ma excitation current could be sourced. The tri-axial accelerometers data sheet states that the device uses 35 ma of excitation current. Connecting these devices caused startup and run-time faults in the Flex-DAC that made their use impossible. The excitation is then set = 0 volts for such a fault. First, it was found that the start-up inrush current failed most of the time. Plugging in the transducers after FlexDAC startup would work when the accelerometers were not outputting significant signals. At that time, an over-current excitation fault would occur. After modifying the cables to insert a 100 ohm resistor the 35 ma current was reduced to 25 ma total current. The tri-axial accelerometer 35 ma specification comes from a 10 VDC excitation specification. The tri-axial accelerometer will operate from 6 VDC to 24 VDC. Therefore, using the extra resistor seems to have solved the problem. The DUC gain is larger, of course, but that is not a problem at all. NCREE felt that it was better to use the resistor and use the 10 VDC standard excitation level so that the tri-axial accelerometer would not be different from other device excitations.